43-STM32Fxxx_MPU9250_EKF

Software layers, function calls and data exchange

Attitude estimation (roll, pitch, yaw, gyro bias) with an MPU9250 9-axis IMU, an Extended Kalman Filter and an SSD1306 OLED display on STM32Fxxx, built on the TM STM32Fxxx HAL libraries.

| Item | Value |
| --- | --- |
| Project folder | 43-STM32Fxxx_MPU9250 _EKF in stm32_utilities |
| Reference target | NUCLEO-F411RE, STM32F411 Cortex-M4F at 100 MHz (HSE 8 MHz, PLLN 400, PLLP 4) |
| Toolchain | Keil uVision 5 (MDK-ARM), optimisation -O1 |
| Author | Fouad El khiati |
| Document version | 1.0, 17 October 2026 |

# Contents

# 1. Introduction

## 1.1 Purpose

This document describes how the firmware in 43-STM32Fxxx_MPU9250 _EKF is split into software layers, which functions each layer calls in the layer below, and what data crosses each boundary: the structures, buffers, units and return codes. It is meant as a reference for maintaining the project, porting it to another MCU or sensor, and reviewing changes.

## 1.2 Source files

| File | Origin | Role |
| --- | --- | --- |
| User/main.c | Project | Board bring-up, filter configuration, main loop, display formatting |
| User/imu_port.c/.h | Project | Hardware adaptation layer: MPU9250 access, unit conversion, magnetometer calibration lifecycle |
| User/ekf.c/.h | Project | Multiplicative attitude EKF, 6 error states (3 attitude, 3 gyro bias). No platform dependency |
| User/magcal.c/.h | Project | Hard/soft-iron magnetometer calibration by least-squares ellipsoid fit |
| User/Gauss_elimination.c/.h | Project | Dense linear solver used by magcal (pivoting, determinant, back substitution) |
| User/defines.h | Project | Compile-time configuration for the TM libraries (NUCLEO_F411) |
| User/stm32fxxx_it.c | Project | Interrupt handlers, including SysTick_Handler |
| tm_stm32_mpu9250, tm_stm32_ssd1306, tm_stm32_fonts, tm_stm32_disco | 00-STM32_LIBRARIES | Device drivers |
| tm_stm32_i2c, tm_stm32_delay, tm_stm32_rcc, tm_stm32_gpio | 00-STM32_LIBRARIES | Peripheral libraries |
| stm32f4xx_hal_*.c, CMSIS startup | 00-HAL_DRIVERS | ST HAL and register-level support |

## 1.3 Execution model

The firmware is a single-threaded super-loop with no RTOS. Every call described in this document is synchronous and blocking: an I2C transfer returns only when the bytes are on the wire or the 1000 ms HAL timeout expires. The only asynchronous path is the 1 ms SysTick interrupt, which advances the time base read by TM_DELAY_Time() and HAL_GetTick() (section 8).

# 2. Layered architecture

The code is organised in five software layers above the hardware. A layer calls only functions of the layers below it; data comes back up through return values and through structures that the caller passes by pointer.

Figure 1. Software layers and call dependencies

| Layer | Modules | Responsibility | Knows about |
| --- | --- | --- | --- |
| L4 Application | main.c | Startup order, filter tuning, loop pacing, display formatting (fixed-point, no stdio) | L3 public APIs; TM display, LED and delay APIs |
| L3 Adaptation | imu_port.c | Turns MPU9250 registers into SI units in imu_sample_t; measures dt; owns the calibration lifecycle | TM MPU9250 and delay; magcal |
| L3 Algorithms | ekf.c, magcal.c, Gauss_elimination.c | Attitude estimation, ellipsoid fit, linear algebra | Only math.h and string.h. Compiles on a PC |
| L2 Device drivers | tm_stm32_mpu9250, tm_stm32_ssd1306 + tm_stm32_fonts, tm_stm32_disco | Register maps, sensor scaling, frame buffer and font rendering, LED | TM I2C, delay, GPIO |
| L1 Peripheral libraries | tm_stm32_i2c, tm_stm32_delay, tm_stm32_rcc, tm_stm32_gpio | I2C handles and pin packs, time base, clock tree, pins | ST HAL |
| L0 HAL + CMSIS | stm32f4xx_hal_i2c, _rcc, _gpio, _cortex, startup | Register access, peripheral state machines, SysTick | Hardware |

## 2.1 Layering rules

- imu_port.c is the only project file that includes tm_stm32_mpu9250.h. Changing the IMU or the MCU means rewriting this one file.

- ekf.c, magcal.c and Gauss_elimination.c include no STM32 header and can be compiled and unit-tested on a PC.

- main.c never touches the sensor. It receives a finished imu_sample_t and hands arrays of float to the filter.

- Display and LED access stay in main.c. No algorithm module draws or blinks.

# 3. Data exchanged between layers

Table 1 lists every structure or buffer that carries data across a layer boundary, who owns its storage, who writes it and who reads it.

| Data | Defined in / storage | Written by | Read by | Contents and units |
| --- | --- | --- | --- | --- |
| imu_sample_t sample | imu_port.h / static in main.c | imu_port_read() | main() → ekf_*() | gyro[3] rad/s, acc[3] g, mag[3] (unit vector if calibrated, else 0.1 µT counts), dt s, mag_valid 0/1 |
| ekf_cfg_t cfg | ekf.h / local in main() | main() | ekf_init() (copied) | gyro_noise, bias_noise, accel_noise, mag_noise, mag_ref[3] |
| ekf_t ekf | ekf.h / static in main.c | ekf_init, ekf_predict, ekf_update_* | ekf_euler(); main() reads ekf.bias[] directly | q[4] quaternion w,x,y,z body→nav; bias[3] rad/s; P[36] 6×6 covariance; cfg copy (200 bytes) |
| magcal_t cal | magcal.h / static in imu_port.c | magcal_reset/add/solve | magcal_apply(), imu_port_read() | ata[36], atb[6] double accumulators; count; offset[3], scale[3]; valid (368 bytes) |
| TM_MPU9250_t dev | tm_stm32_mpu9250.h / static in imu_port.c | TM_MPU9250_Init/Read*() | imu_port.c | Ax..Az g, Gx..Gz °/s, Mx_Raw..Mz_Raw int16, multipliers AMult, GMult, MMult, I2C addresses |
| float A[36], aug[36], v[6] | locals in magcal_solve() | magcal_solve, then Gauss functions in place | magcal_solve() | Normal matrix (destroyed), identity → inverse, right-hand side → solution |
| SSD1306_Buffer[1024] | static in tm_stm32_ssd1306.c | Fill, Puts, DrawPixel | TM_SSD1306_UpdateScreen() | 128×64 monochrome frame, 8 pages of 128 bytes |
| char buf[24] | local in draw_line() | fmt(), putu() | TM_SSD1306_Puts() | ASCII such as R:-12.34 |
| TM_Time (uint32_t) | tm_stm32_delay.c | HAL_IncTick() from SysTick ISR | TM_DELAY_Time(), HAL_GetTick() | Milliseconds since startup |
| I2C_HandleTypeDef | tm_stm32_i2c.c (one per I2Cx) | TM_I2C_Init() | every TM_I2C_* call | HAL state of I2C1; shared by the MPU9250 and the SSD1306 |

Table 1. Data crossing layer boundaries

## 3.1 Passing conventions

- Structures are passed by pointer and filled in place. No function allocates memory; every buffer is static or on the stack.

- Vectors are float[3] in X, Y, Z order. Matrices are row-major float arrays (P[i*6 + j]).

- Status comes back as int (1 = success, 0 = rejected or failed) in the project layers, and as enum result codes (TM_I2C_Result_t, TM_MPU9250_Result_t, HAL_StatusTypeDef) in the libraries.

# 4. Function interfaces by layer

For every function the tables give its caller, inputs, outputs, and the functions it calls in the layer below.

## 4.1 L4: main.c

| Function | Inputs | Outputs / effect | Calls |
| --- | --- | --- | --- |
| main(void) | none | never returns | startup chain (section 5), loop chain (section 6) |
| draw_line(y, label, value) | row in pixels, label string, value in degrees | text in the frame buffer | fmt, TM_SSD1306_GotoXY, TM_SSD1306_Puts |
| fmt(dst, label, val) | float value | dst = label + signed value with 2 decimals; clamps to 9999, handles NaN/inf | putu |
| putu(p, v) | uint32_t | decimal digits written at p; returns the new end pointer | none |
| fail_blink(void) | none | toggles the LED every 80 ms forever | TM_DISCO_LedToggle, Delayms |

## 4.2 L3: imu_port.c

| Function | Called by | Inputs → outputs | Calls |
| --- | --- | --- | --- |
| imu_port_init(void) | main | → 1 if the MPU9250 answered, else 0. Resets cal, stores last_ms | TM_MPU9250_Init, magcal_reset, TM_DELAY_Time |
| imu_port_read(imu_sample_t *s) | main (every loop) | → fills *s; returns 1 if 0 < dt < 0.5 s | TM_DELAY_Time, TM_MPU9250_ReadAcce/ReadGyro/ReadMag, read_mag_raw, magcal_apply |
| read_mag_raw(out[3]) (static) | imu_port_read, imu_port_cal_feed | dev.M*_Raw × dev.MMult → out in 0.1 µT | none (reads dev) |
| imu_port_cal_begin(void) | application (not wired yet) | clears the accumulators | magcal_reset |
| imu_port_cal_feed(void) | application (not wired yet) | → running sample count | TM_MPU9250_ReadMag, read_mag_raw, magcal_add |
| imu_port_cal_finish(void) | application (not wired yet) | → 1 if the fit succeeded; from then on imu_port_read applies it | magcal_solve |

## 4.3 L3: ekf.c

| Function | Inputs | Outputs / state changes | Internal calls |
| --- | --- | --- | --- |
| ekf_init(f, cfg) | ekf_cfg_t* | q = identity, bias = 0, P = diag(0.1, 0.1, 0.1, 0.01, 0.01, 0.01), cfg copied | memset |
| ekf_predict(f, gyro, dt) | gyro[3] rad/s, dt s | q integrated with bias-corrected rate; P = Φ P Φᵀ + Q | quat_mul, quat_normalize, memcpy |
| ekf_update_accel(f, acc) | acc[3] g | returns 0 if ¦acc¦ is outside 0.85–1.15 g; otherwise corrects q, bias, P toward gravity [0, 0, 1] | vector_update |
| ekf_update_mag(f, mag) | mag[3] calibrated | corrects q, bias, P toward cfg.mag_ref | vector_update |
| ekf_euler(f, &r, &p, &y) | ekf_t* | roll, pitch, yaw in rad (NULL pointers skipped) | atan2f, asinf |
| vector_update(...) (static) | measurement, nav reference, noise | innovation y = z − Rᵀref; gain K = P Hᵀ S⁻¹; multiplicative correction; P −= K H P; re-symmetrise | normalize3, quat_to_R, R_transpose_mul, inv3x3, quat_mul, quat_normalize |

## 4.4 L3: magcal.c and Gauss_elimination.c

| Function | Inputs | Outputs / state changes | Calls |
| --- | --- | --- | --- |
| magcal_reset(c) | magcal_t* | zeroes the accumulators; scale = 1 | memset |
| magcal_add(c, mx, my, mz) | raw sample | adds row [y² z² x y z 1] and target −x² (scaled by 1/50) into ata, atb; count++ | none |
| magcal_solve(c) | accumulated sums | → 1 and sets offset, scale, valid; → 0 if count < 100, singular, or not an ellipsoid | determinant, inversion, sqrtf |
| magcal_apply(c, raw, out) | raw float[3] | out = (raw − offset) × scale | none |
| determinant(a, b, tb, n) | n×n a, identity b, RHS tb | a made upper-triangular; same row operations on b and tb; returns det or 0 | partial_pivot → ligne_swap → swap; matrix_elimination |
| inversion(a, b, tb, n) | output of determinant | b = A⁻¹, tb = solution x | none |

## 4.5 L2 and L1: TM libraries used

| Function | Inputs → outputs | Calls down to |
| --- | --- | --- |
| TM_MPU9250_Init(&dev, Device_0) | → Result_Ok / DeviceNotConnected; sets AMult = 2/32768, GMult = 250/32768, MMult = 10·4912/32768 | TM_DELAY_Init, TM_I2C_Init, TM_I2C_IsDeviceConnected, TM_I2C_Read, TM_I2C_Write, Delayms |
| TM_MPU9250_ReadAcce(&dev) | 6 bytes from 0x3B → Ax_Raw.., Ax..Az in g | TM_I2C_ReadMulti |
| TM_MPU9250_ReadGyro(&dev) | 6 bytes from 0x43 → Gx_Raw.., Gx..Gz in °/s | TM_I2C_ReadMulti |
| TM_MPU9250_ReadMag(&dev) | ST1 from AK8963; if data ready, 7 bytes from 0x03 → Mx_Raw..Mz_Raw (kept if overflow) | TM_I2C_Read, TM_I2C_ReadMulti |
| TM_SSD1306_Init() | → 1 / 0; sends the init command sequence | TM_DELAY_Init, TM_I2C_Init, TM_I2C_IsDeviceConnected, TM_I2C_Write |
| TM_SSD1306_Fill, GotoXY, Puts | colour, cursor, string + TM_Font_7x10 → frame buffer | TM_SSD1306_Putc → TM_SSD1306_DrawPixel (RAM only) |
| TM_SSD1306_UpdateScreen() | frame buffer → display | 8 × (3 × TM_I2C_Write + TM_I2C_WriteMulti 128 B) |
| TM_DISCO_LedInit/On/Toggle | LED id | TM_GPIO_* → HAL_GPIO_* |
| TM_I2C_Init(I2C1, PinsPack_1, 400000) | PB6/PB7 alternate function, 400 kHz | TM_GPIO_InitAlternate, HAL_I2C_Init |
| TM_I2C_Read, TM_I2C_ReadMulti | device, register, buffer, count → TM_I2C_Result_t | HAL_I2C_Master_Transmit (register) then HAL_I2C_Master_Receive (data) |
| TM_I2C_Write | device, register, 1 byte | HAL_I2C_Master_Transmit (2 bytes) |
| TM_I2C_WriteMulti | device, register, buffer, count | HAL_I2C_Mem_Write |
| TM_I2C_IsDeviceConnected | device address | HAL_I2C_IsDeviceReady |
| TM_DELAY_Init, Delayms, TM_DELAY_Time | ms | DWT counter enable; HAL_Delay; reads TM_Time |
| TM_RCC_InitSystem | PLL defines from the Keil target | HAL_RCC_OscConfig, HAL_RCC_ClockConfig |

# 5. Startup sequence

The order is fixed. TM_DELAY_Init must run before imu_port_init because imu_port takes its time base from TM_DELAY_Time. The SSD1306 is initialised before the MPU9250 because both share I2C1.

| # | Call chain (top → bottom layer) | Data exchanged | Purpose |
| --- | --- | --- | --- |
| 1 | main → TM_RCC_InitSystem → HAL_RCC_OscConfig / ClockConfig | PLL constants (compile-time) | SYSCLK = 100 MHz |
| 2 | main → HAL_Init → SysTick configuration | none | 1 ms SysTick interrupt |
| 3 | main → TM_DELAY_Init | none | enables the DWT cycle counter; TM_Time starts counting |
| 4 | main → TM_DISCO_LedInit → TM_GPIO_Init → HAL_GPIO_Init | LED pin PA5 | status LED |
| 5 | main → Delayms(300) → HAL_Delay | 300 ms | power-up time for both modules |
| 6 | main → TM_SSD1306_Init → TM_I2C_Init → HAL_I2C_Init; then TM_I2C_IsDeviceConnected and init commands via TM_I2C_Write | ← 1 / 0 | brings up I2C1 and the display. On 0: fail_blink() |
| 7 | main → TM_SSD1306_Fill/GotoXY/Puts/UpdateScreen | "Starting..." | user feedback |
| 8 | main → imu_port_init → TM_MPU9250_Init → TM_I2C_* → HAL_I2C_* | ← 1 / 0 | WHO_AM_I check (0x71), wake-up, DLPF 41 Hz, ODR 200 Hz, ±250 °/s, ±2 g, AK8963 16-bit at 8 Hz. On 0: "MPU9250 FAIL" and halt |
| 9 | imu_port_init → magcal_reset(&cal); last_ms = TM_DELAY_Time() | magcal_t, uint32_t | calibration inactive; first dt reference |
| 10 | main fills ekf_cfg_t cfg → ekf_init(&ekf, &cfg) | ekf_cfg_t copied into ekf_t | level attitude, zero bias, initial covariance |
| 11 | main → TM_DISCO_LedOn | none | steady LED = running |

# 6. Main loop: call chains and data flow

Figure 2 shows one iteration. The steps below follow the same order and give the full depth of each call chain.

Figure 2. One iteration of the main loop

## 6.1 Sensor acquisition

main()

└─ imu_port_read(&sample)                                         [L3]

├─ TM_DELAY_Time()                     → now (ms)               [L1]

├─ TM_MPU9250_ReadAcce(&dev)                                    [L2]

│   └─ TM_I2C_ReadMulti(I2C1, 0xD0, 0x3B, data, 6)              [L1]

│       ├─ HAL_I2C_Master_Transmit(reg 0x3B)                    [L0]

│       └─ HAL_I2C_Master_Receive(6 bytes)                      [L0]

├─ TM_MPU9250_ReadGyro(&dev)        same chain, register 0x43

├─ TM_MPU9250_ReadMag(&dev)

│   ├─ TM_I2C_Read(0x18, ST1)       → data-ready bit

│   └─ TM_I2C_ReadMulti(0x18, 0x03, 7 bytes)   only if ready

├─ read_mag_raw(raw)                → dev.M*_Raw × MMult

├─ magcal_apply(&cal, raw, s->mag)  only if cal.valid

└─ return (0 < dt < 0.5 s)

Data returned to main in sample: gyro = dev.G* × DEG2RAD (rad/s), acc = dev.A* (g), mag = calibrated vector or raw values, dt = (now − last_ms) × 0.001 s, mag_valid. If imu_port_read returns 0 the iteration skips the filter and the display.

## 6.2 Estimation

main()

├─ ekf_predict(&ekf, sample.gyro, sample.dt)

│   ├─ quat_mul(q, dq) → quat_normalize(q)       attitude propagation

│   └─ Φ·P·Φᵀ + Q  (two 6×6×6 products)          covariance propagation

├─ ekf_update_accel(&ekf, sample.acc)            returns 0 if |acc| ∉ [0.85, 1.15] g

│   └─ vector_update(f, acc, {0,0,1}, accel_noise)

│       ├─ normalize3 → quat_to_R → R_transpose_mul   predicted gravity in body frame

│       ├─ H, P·Hᵀ, S = H·P·Hᵀ + R → inv3x3(S)          3×3 innovation covariance

│       ├─ K = P·Hᵀ·S⁻¹,  dx = K·(z − h)

│       ├─ quat_mul(q, δq) → quat_normalize;  bias += dx[3..5]

│       └─ P −= K·H·P, re-symmetrise

├─ ekf_update_mag(&ekf, sample.mag)              compiled only if EKF_USE_MAG = 1

└─ ekf_euler(&ekf, &roll, &pitch, &yaw)          → radians

The filter exchanges data only through its arguments and the ekf_t structure. Nothing in ekf.c reads a global or calls a library function apart from math.h, memset and memcpy.

## 6.3 Display

main()

├─ TM_SSD1306_Fill(BLACK)                        memset of the 1024-byte buffer

├─ draw_line(y, "R:", roll × RAD2DEG)  × 6 lines (R, P, Y, bx, by, bz)

│   ├─ fmt() → putu()                           float → "R:-12.34" (no stdio)

│   ├─ TM_SSD1306_GotoXY(0, y)

│   └─ TM_SSD1306_Puts(buf, &TM_Font_7x10)

│       └─ Putc → DrawPixel                     70 pixels per character, RAM only

└─ TM_SSD1306_UpdateScreen()

└─ for page 0..7:

├─ TM_I2C_Write ×3 (0xB0+page, 0x00, 0x10)  → HAL_I2C_Master_Transmit

└─ TM_I2C_WriteMulti(0x40, 128 bytes)        → HAL_I2C_Mem_Write

The bias lines read ekf.bias[] directly from the filter structure. This is the only place where main.c reaches inside a lower-layer structure instead of going through a function.

## 6.4 Loop pacing

Delayms(10) → HAL_Delay(10) busy-waits on HAL_GetTick(). The real loop period is the sum of all the steps above (section 9), not 10 ms.

# 7. Magnetometer calibration path

The calibration functions are implemented and exported by imu_port.h, but main.c does not call them yet. The intended sequence, and the full call chain it triggers, is:

imu_port_cal_begin()

└─ magcal_reset(&cal)                          zero ata[36], atb[6], count

repeat while the board is rotated in all orientations:

imu_port_cal_feed()                             → returns cal.count

├─ TM_MPU9250_ReadMag(&dev) → TM_I2C_Read / ReadMulti → HAL_I2C_*

├─ read_mag_raw(raw)

└─ magcal_add(&cal, raw[0], raw[1], raw[2])   fold one row into the 6×6 normal equations

imu_port_cal_finish()                           → 1 on success

└─ magcal_solve(&cal)

├─ copy double ata/atb → float A[36], v[6]; aug = identity

├─ determinant(A, aug, v, 6)

│   ├─ partial_pivot → ligne_swap → swap    row swaps on A, aug and v

│   └─ matrix_elimination                  zero below each pivot

├─ inversion(A, aug, v, 6)                 back substitution → v = [B C P Q R D]

└─ centre (x0, y0, z0), K, offset[3], scale[3], valid = 1

Once cal.valid is 1, every following imu_port_read() routes the raw magnetometer vector through magcal_apply() and sets sample.mag_valid = 1. Only then should EKF_USE_MAG be set to 1 so that main calls ekf_update_mag().

# 8. Time base and interrupt path

The SysTick interrupt is the only asynchronous data path. The TM delay library overrides two weak HAL functions so that the HAL and the TM libraries share one counter:

SysTick (1 ms) → SysTick_Handler() [stm32fxxx_it.c]

└─ HAL_IncTick()   overridden in tm_stm32_delay.c → TM_Time++

HAL_GetTick()     overridden in tm_stm32_delay.c → returns TM_Time

TM_DELAY_Time()   macro → TM_Time

- imu_port_read() reads TM_Time to compute dt with a resolution of 1 ms. The rounding errors cancel over successive samples, so the integrated time stays exact.

- HAL_Delay(), and every HAL I2C timeout, are measured against the same counter.

# 9. Unit conversions along the data path

| Stage | Layer | Variable | Type / unit | Conversion |
| --- | --- | --- | --- | --- |
| Register read | L0/L1 | data[6] | uint8_t, big-endian (MPU) / little-endian (AK8963) | bytes from the bus |
| Driver raw | L2 | Ax_Raw, Gx_Raw, Mx_Raw | int16_t counts | (hi << 8) ¦ lo |
| Driver scaled | L2 | Ax..Az | float g | × AMult = 2/32768 (±2 g range) |
| Driver scaled | L2 | Gx..Gz | float °/s | × GMult = 250/32768 (±250 °/s range) |
| Port | L3 | sample.gyro | float rad/s | × DEG2RAD = 0.017453292 |
| Port | L3 | sample.acc | float g | unchanged |
| Port | L3 | raw magnetometer | float, 0.1 µT | Mx_Raw × MMult = 10 × 4912/32768 (≈ 1.5 per count) |
| Port | L3 | sample.mag | float, ≈ unit length | (raw − offset) × scale when calibrated |
| Port | L3 | sample.dt | float s | (now − last_ms) × 0.001 |
| Filter | L3 | q, bias | float, rad/s for bias | measurements normalised internally |
| Filter output | L3 | roll, pitch, yaw | float rad | ekf_euler() |
| Application | L4 | displayed value | float °, then fixed point ×100 | × RAD2DEG = 57.29578, then fmt() |
| Display | L2 | frame buffer | bits, 1 per pixel | TM_Font_7x10 glyphs |

# 10. Time spent in each layer

Estimated from the bus protocol at 400 kHz (22.5 µs per byte including ACK) and from the operation count of the filter at 100 MHz with -O1. These are calculated figures, not measurements on hardware.

| Step | Layers involved | Time per loop | Share |
| --- | --- | --- | --- |
| TM_SSD1306_UpdateScreen() | L2 → L1 → L0 → I2C | ≈ 25 ms | ≈ 68 % |
| Delayms(10) | L1 → L0 | 10 ms | ≈ 27 % |
| Text rendering (Fill, Puts) | L2, RAM only | ≈ 0.5–1 ms | ≈ 2 % |
| imu_port_read() (3 sensor reads) | L3 → L2 → L1 → L0 → I2C | ≈ 0.5–0.7 ms | ≈ 2 % |
| ekf_predict + ekf_update_accel + ekf_euler | L3 only | ≈ 70 µs | < 0.2 % |
| Total loop period |  | ≈ 37 ms (≈ 27 Hz) | 100 % |

The display transfer through the I2C stack sets the filter rate. The estimation layer itself uses a negligible share of the CPU.

# 11. Observations at layer boundaries

| Boundary | Observation | Consequence | Suggested change |
| --- | --- | --- | --- |
| L2 → L3 (MPU9250 driver) | TM_MPU9250_ReadAcce/Gyro/Mag are declared to return TM_MPU9250_Result_t but have no return statement | imu_port cannot detect a failed read; stale values look valid | Add return statements and propagate status in imu_port_read |
| L1 → L2 (I2C) | TM_I2C_* error results are ignored by the drivers | a bus fault can block up to 1 s per call, silently | Check results; add bus recovery |
| L2 → L3 (magnetometer) | ReadMag writes only M*_Raw; Mx/My/Mz stay empty | worked around by read_mag_raw() in imu_port | Fix the driver, or keep the workaround documented |
| L3 (imu_port) | AK8963 axes are not aligned with the accel/gyro axes, and no remapping is done | a wrong heading correction once EKF_USE_MAG = 1 | Remap mag X↔Y and negate Z in imu_port_read |
| L3 → L4 | imu_sample_t has no flag telling whether the magnetometer sample is new (AK8963 at 8 Hz) | the same reading would be fused several times | Add a mag_new flag |
| L4 → L3 | main.c reads ekf.bias[] directly | couples the application to the filter's internal layout | Add ekf_bias() accessor |
| L4 → L2 (display) | A full-screen update every loop | limits the filter to ≈ 27 Hz | Refresh at 5–10 Hz, or use tm_stm32_i2c_dma |
| L1 (I2C1 sharing) | TM_MPU9250_Init calls TM_I2C_Init again on the bus the display already set up | harmless today; breaks if the pin packs differ | Keep both on TM_I2C_PinsPack_1 in defines.h |